free5GC Deployment on MicroK8s

Complete Step-by-Step Troubleshooting Guide  |  April 2026

# SECTION 1: System Requirements & Prerequisites

| Requirement | Specification |
| --- | --- |
| OS | Ubuntu 20.04 or 22.04 LTS (bare metal or VM) |
| Min CPU | 4 vCPUs |
| Min RAM | 8 GB |
| Disk | 40 GB minimum |
| Kernel | 5.4 or newer (check with: uname -r) |
| Network | Know your main interface (run: ip a) |
| VM Platform | VirtualBox with Host-only Adapter recommended |

⚠️  CRITICAL: If using VirtualBox with Bridged WiFi adapter, your IP will change and break everything. Set up a Host-only Adapter for a stable IP first.

# SECTION 2: VirtualBox Network Setup (IMPORTANT - Do This First)

If running Ubuntu in VirtualBox, configure a Host-only Adapter to prevent IP changes that break MicroK8s certificates.

## Step 2.1 - Add Host-only Adapter in VirtualBox GUI

- Shut down your VM completely

- Open VirtualBox → Go to File → Host Network Manager

- Create a host-only network if none exists (e.g. vboxnet0, range 192.168.56.0/24)

- Go to VM Settings → Network → Adapter 2

- Enable it → Attached to: Host-only Adapter → Select vboxnet0

- Click OK → Start the VM

## Step 2.2 - Configure Static IP on Host-only Interface

Find the new interface inside Ubuntu:

ip a   # look for new interface, e.g. enp0s8

Create the netplan config file:

sudo nano /etc/netplan/01-hostonly-static.yaml

Paste this content (adjust interface name if needed):

network:

version: 2

ethernets:

enp0s8:                    # your host-only interface

dhcp4: no

addresses:

- 192.168.56.101/24    # static IP - never changes

Fix permissions and apply:

sudo chmod 600 /etc/netplan/01-hostonly-static.yaml

sudo chmod 600 /etc/netplan/01-network-manager-all.yaml

sudo netplan apply

Verify the IP is assigned:

ip a show enp0s8

💡  From now on, connect MobaXterm to 192.168.56.101 - this IP will never change regardless of reboots or WiFi drops.

# SECTION 3: Install gtp5g Kernel Module (CRITICAL - Do This Before MicroK8s)

⚠️  This is the #1 reason free5GC UPF fails silently. The gtp5g module MUST be installed on the HOST before deploying.

Install from your HOME directory, NOT from inside the Helm chart folder:

cd ~

sudo apt -y update

sudo apt -y install git gcc g++ cmake autoconf libtool pkg-config libmnl-dev libyaml-dev

git clone https://github.com/free5gc/gtp5g.git

cd gtp5g

make clean && make

sudo make install

Verify the module loaded successfully:

lsmod | grep gtp5g

Expected output:

gtp5g    151552  0

udp_tunnel    32768  2 gtp5g,vxlan

If nothing shows, load it manually:

sudo modprobe gtp5g

lsmod | grep gtp5g

⚠️  Do NOT install gtp5g from inside ~/free5gc-helm/charts/free5gc/gtp5g - that is just bundled for reference, not for installation.

# SECTION 4: Install MicroK8s, kubectl, and Helm

sudo snap install microk8s --classic --channel=1.28/stable

sudo snap install kubectl --classic

sudo snap install helm --classic

## Step 4.1 - Set up user permissions

sudo groupadd microk8s 2>/dev/null || true

sudo usermod -aG microk8s $USER

newgrp microk8s

## Step 4.2 - Configure kubectl to use MicroK8s

mkdir -p ~/.kube

chmod 0700 ~/.kube

microk8s config > ~/.kube/config

## Step 4.3 - Wait for MicroK8s to be ready

microk8s status --wait-ready

microk8s kubectl get nodes

# SECTION 5: Kernel Networking Configuration

## Step 5.1 - Apply sysctl settings (PERSISTENT across reboots)

cat <<EOF | sudo tee /etc/sysctl.d/k8s.conf

net.bridge.bridge-nf-call-iptables = 1

net.bridge.bridge-nf-call-ip6tables = 1

net.ipv4.ip_forward = 1

EOF

sudo sysctl --system

What these settings do:

- bridge-nf-call-iptables: Makes bridged pod traffic go through iptables rules (required for Kubernetes service routing)

- bridge-nf-call-ip6tables: Same for IPv6 traffic

- ip_forward: Allows kernel to route packets between interfaces (CRITICAL for UPF data plane)

## Step 5.2 - Configure kubelet for IP forwarding

echo '--allowed-unsafe-sysctls "net.ipv4.ip_forward"' | sudo tee -a /var/snap/microk8s/current/args/kubelet

## Step 5.3 - IMPORTANT: Check your CNI (Calico vs Flannel)

ls /var/snap/microk8s/current/args/cni-network/

If you see flannel.conflist → You have Flannel CNI (IP forwarding works by default, skip cni.yaml editing)

If you see cni.yaml → You have Calico CNI, edit it to add:

"container_settings": {

"allow_ip_forwarding": true

}

Restart MicroK8s after any config changes:

microk8s stop && microk8s start

microk8s status --wait-ready

# SECTION 6: Enable Required MicroK8s Addons

⚠️  You must enable 'community' BEFORE enabling 'multus' - otherwise multus addon won't be found.

microk8s enable community

microk8s enable multus

microk8s enable dns

microk8s enable hostpath-storage

Wait for all addons to be ready:

microk8s kubectl get pods -A   # wait until all show Running

# SECTION 7: Create Storage Directories and Persistent Volumes

## Step 7.1 - Create host directories

mkdir -p /home/sarang/free5gc-storage/mongo

mkdir -p /home/sarang/free5gc-storage/cert

sudo chmod 777 /home/sarang/free5gc-storage/mongo

sudo chmod 777 /home/sarang/free5gc-storage/cert

💡  These are just EMPTY storage directories. MongoDB will write into them automatically. Do NOT put any files in them manually.

## Step 7.2 - Get your node name

kubectl get nodes   # note the NAME column

## Step 7.3 - Create pv-mongo.yaml

nano ~/pv-mongo.yaml

Content (replace YOUR_USER and YOUR_NODE_NAME):

apiVersion: v1

kind: PersistentVolume

metadata:

name: free5gc-pv-mongo

labels:

project: free5gc

spec:

capacity:

storage: 8Gi

accessModes:

- ReadWriteOnce

persistentVolumeReclaimPolicy: Retain

storageClassName: microk8s-hostpath

local:

path: /home/sarang/free5gc-storage/mongo

nodeAffinity:

required:

nodeSelectorTerms:

- matchExpressions:

- key: kubernetes.io/hostname

operator: In

values:

- sarang   # YOUR node name here

## Step 7.4 - Create pv-cert.yaml

nano ~/pv-cert.yaml

apiVersion: v1

kind: PersistentVolume

metadata:

name: free5gc-pv-cert

labels:

project: free5gc

spec:

capacity:

storage: 2Mi

accessModes:

- ReadOnlyMany

persistentVolumeReclaimPolicy: Retain

storageClassName: microk8s-hostpath

local:

path: /home/sarang/free5gc-storage/cert

nodeAffinity:

required:

nodeSelectorTerms:

- matchExpressions:

- key: kubernetes.io/hostname

operator: In

values:

- sarang   # YOUR node name here

## Step 7.5 - Apply PVs

kubectl apply -f ~/pv-mongo.yaml

kubectl apply -f ~/pv-cert.yaml

kubectl get pv   # both should show 'Available'

# SECTION 8: Clone Helm Chart and Deploy free5GC

## Step 8.1 - Clone the Helm chart

cd ~

git clone https://github.com/free5gc/free5gc-helm.git

cd free5gc-helm/charts

## Step 8.2 - Find your network interface name

ip a   # look for main interface, e.g. enp0s3, eth0, enp0s8

## Step 8.3 - Create the namespace

kubectl create ns free5gc

## Step 8.4 - Pre-pull the MongoDB image (IMPORTANT)

⚠️  The MongoDB image bitnamilegacy/mongodb:7.0.9-debian-12-r4 must be pulled manually first to avoid silent failures.

sudo microk8s ctr images pull docker.io/bitnamilegacy/mongodb:7.0.9-debian-12-r4

Verify it pulled:

sudo microk8s ctr images list | grep bitnamilegacy

## Step 8.5 - Install the Helm chart

⚠️  Run this from ~/free5gc-helm/charts/ directory, NOT from inside the free5gc/ subfolder.

cd ~/free5gc-helm/charts

pwd   # verify you see 'charts' in the path, and 'ls' shows free5gc/ and ueransim/ folders

helm install -n free5gc free5gc-helm ./free5gc/ \

--set global.n2network.masterIf=enp0s3 \

--set global.n3network.masterIf=enp0s3 \

--set global.n4network.masterIf=enp0s3 \

--set global.n6network.masterIf=enp0s3 \

--set global.n9network.masterIf=enp0s3

Replace enp0s3 with your actual interface name.

# SECTION 9: Troubleshooting Guide (Problems We Encountered)

## Problem 1: kubectl logs TLS Certificate Error

Error: x509: certificate is valid for 192.168.2.54, not 192.168.39.54

Root cause: IP changed (DHCP/VM restart) but MicroK8s certs were generated for old IP.

Fix - Add IPs to cert template and regenerate:

sudo nano /var/snap/microk8s/current/certs/csr.conf.template

Add your IPs in the [alt_names] section:

IP.3 = 192.168.39.54

IP.4 = 192.168.56.101

#MOREIPS

Regenerate kubelet cert manually:

microk8s stop

sudo rm -f /var/snap/microk8s/current/certs/kubelet.crt

sudo rm -f /var/snap/microk8s/current/certs/kubelet.key

sudo rm -f /var/snap/microk8s/current/certs/csr.conf.rendered

sudo openssl genrsa -out /var/snap/microk8s/current/certs/kubelet.key 2048

sudo openssl req -new -key /var/snap/microk8s/current/certs/kubelet.key \

-out /var/snap/microk8s/current/certs/kubelet.csr \

-config /var/snap/microk8s/current/certs/csr.conf

sudo openssl x509 -req \

-in /var/snap/microk8s/current/certs/kubelet.csr \

-CA /var/snap/microk8s/current/certs/ca.crt \

-CAkey /var/snap/microk8s/current/certs/ca.key \

-CAcreateserial \

-out /var/snap/microk8s/current/certs/kubelet.crt \

-days 3650 \

-extensions v3_ext \

-extfile /var/snap/microk8s/current/certs/csr.conf

sudo microk8s refresh-certs -e server.crt

microk8s start

microk8s status --wait-ready

microk8s config > ~/.kube/config

Verify the kubelet cert now has correct IPs:

openssl s_client -connect 192.168.39.54:10250 </dev/null 2>/dev/null | \

openssl x509 -text -noout | grep -A5 "Subject Alternative Name"

## Problem 2: MongoDB Stuck in ContainerCreating

Error: MountVolume failed: path does not exist

Root cause: Storage directory not created on host.

mkdir -p /home/sarang/free5gc-storage/mongo

mkdir -p /home/sarang/free5gc-storage/cert

sudo chmod 777 /home/sarang/free5gc-storage/mongo

sudo chmod 777 /home/sarang/free5gc-storage/cert

kubectl delete pod mongodb-0 -n free5gc --force --grace-period=0

## Problem 3: UPF Pods in CrashLoopBackOff (Multus IP Already in Use)

Error: failed to add IP addr 10.100.50.233 to n3: address already in use

Root cause: Hundreds of orphaned CNI namespaces from previous failed attempts.

Find them:

sudo ip netns list

Delete them all:

sudo ip -all netns delete

Force delete any remaining stuck ones:

for ns in $(sudo ls /var/run/netns/); do

sudo umount /var/run/netns/$ns 2>/dev/null

sudo rm -f /var/run/netns/$ns

done

Verify all gone:

sudo ip netns list | wc -l   # should return 0

Restart MicroK8s and force delete UPF pods:

microk8s stop && microk8s start

microk8s status --wait-ready

kubectl delete pod -n free5gc -l app.kubernetes.io/name=free5gc-upf --force --grace-period=0

⚠️  FUTURE PREVENTION: Always run 'helm uninstall free5gc-helm -n free5gc' before retrying a deployment. Never just delete pods individually.

## Problem 4: MongoDB Image Not Pulling (bitnamilegacy)

Issue: MongoDB stuck in ContainerCreating with no events for hours

Root cause: The image docker.io/bitnamilegacy/mongodb:7.0.9-debian-12-r4 silently fails to auto-pull.

Fix: Pre-pull the image manually before deploying:

sudo microk8s ctr images pull docker.io/bitnamilegacy/mongodb:7.0.9-debian-12-r4

This shows a real progress bar. Wait for it to complete, then verify:

sudo microk8s ctr images list | grep bitnamilegacy

## Problem 5: PVC Stuck in Terminating State

kubectl patch pvc datadir-mongodb-0 -n free5gc \

-p '{"metadata":{"finalizers":[]}}' --type=merge

## Problem 6: Helm Install - 'path not found' Error

Error: INSTALLATION FAILED: path ./free5gc/ not found

Root cause: Running helm install from inside the free5gc/ folder instead of from charts/ folder.

cd ~/free5gc-helm/charts   # correct directory

ls   # should show: free5gc/  ueransim/

helm install -n free5gc free5gc-helm ./free5gc/ ...

## Problem 7: Kubernetes API Unreachable

Error: dial tcp 192.168.x.x:16443: i/o timeout

Fix options:

microk8s config > ~/.kube/config   # refresh kubeconfig

microk8s status --wait-ready        # check if running

microk8s helm3 install ...          # use built-in helm instead of system helm

# SECTION 10: Steps to Follow After MongoDB Image Pull Completes

Follow these steps in order once the image pull finishes

## Step 10.1 - Verify image is pulled

sudo microk8s ctr images list | grep bitnamilegacy

## Step 10.2 - Force delete MongoDB pod to trigger fresh start

kubectl delete pod mongodb-0 -n free5gc --force --grace-period=0

## Step 10.3 - Watch MongoDB come up

kubectl get pod mongodb-0 -n free5gc -w

Wait for: 0/1 ContainerCreating → 1/1 Running

💡  This may take 2-3 minutes after image is pulled. Once MongoDB is Running, all other pods will start automatically.

## Step 10.4 - Watch the chain reaction

watch kubectl get pods -n free5gc

Expected startup order:

- mongodb-0 → Running (1/1)

- NRF → Running (after MongoDB)

- AUSF, UDR, UDM → Running (after NRF)

- AMF, SMF, PCF, NSSF, NEF, CHF → Running

- UPF pods (iupf1, psaupf1, psaupf2) → Running (after NRF)

- WebUI → Running

⚠️  UPF pods will keep showing CrashLoopBackOff/Completed until MongoDB+NRF are up. This is normal - ignore them until MongoDB is Running.

## Step 10.5 - Verify all pods are Running

kubectl get pods -n free5gc

All pods should show STATUS: Running and READY: 1/1

## Step 10.6 - Verify UPF IP forwarding

kubectl exec -it -n free5gc deployment/free5gc-helm-free5gc-upf-iupf1 \

-- cat /proc/sys/net/ipv4/ip_forward

Output should be: 1

## Step 10.7 - Access the WebUI

Open in browser:

http://<YOUR_NODE_IP>:30500

Get node IP with:

kubectl get node -o wide

Default WebUI credentials: admin / free5gc

## Step 10.8 - Deploy UERANSIM (optional - for testing)

cd ~/free5gc-helm/charts

helm install -n free5gc ueransim ./ueransim/ \

--set global.n2network.masterIf=enp0s3 \

--set global.n3network.masterIf=enp0s3

## Step 10.9 - End-to-end connectivity test

kubectl exec -it -n free5gc deployment/ueransim-ue \

-- ping -I uesimtun0 8.8.8.8

# SECTION 11: Quick Reference - Common Commands

| Task | Command |
| --- | --- |
| Check all pods | kubectl get pods -n free5gc |
| Watch pods live | watch kubectl get pods -n free5gc |
| Describe a pod | kubectl describe pod <name> -n free5gc ¦ tail -30 |
| Get pod logs | kubectl logs -n free5gc <pod-name> |
| Get crash logs | kubectl logs -n free5gc <pod-name> --previous |
| Check PVCs | kubectl get pvc -n free5gc |
| Check PVs | kubectl get pv |
| Uninstall Helm | helm uninstall free5gc-helm -n free5gc |
| Check gtp5g | lsmod ¦ grep gtp5g |
| Check netns | sudo ip netns list ¦ wc -l |
| Refresh kubeconfig | microk8s config > ~/.kube/config |
| Restart MicroK8s | microk8s stop && microk8s start |
| Check MicroK8s status | microk8s status --wait-ready |
| Pre-pull MongoDB image | sudo microk8s ctr images pull docker.io/bitnamilegacy/mongodb:7.0.9-debian-12-r4 |
| Delete all netns | sudo ip -all netns delete |
| Force delete pod | kubectl delete pod <name> -n free5gc --force --grace-period=0 |
| Clean fresh start | helm uninstall free5gc-helm -n free5gc && kubectl delete namespace free5gc && kubectl create namespace free5gc |

# SECTION 12: Important Notes & Lessons Learned

## Always do a clean uninstall before retrying

helm uninstall free5gc-helm -n free5gc

kubectl delete namespace free5gc

sudo ip -all netns delete

kubectl create namespace free5gc

Never just delete individual pods and retry - this leaves orphaned CNI namespaces.

## Correct startup dependency chain

MongoDB → NRF → Core NFs (AMF/SMF/etc.) → UPF → UERANSIM

If any early component fails, everything downstream will also fail. Always fix from the bottom up.

## Static IP is mandatory for stability

A changing IP breaks MicroK8s TLS certificates. Always configure a static IP (either via netplan or VirtualBox Host-only Adapter) before deploying.

## Pre-pull large images

Always pre-pull large images manually using microk8s ctr images pull to avoid silent failures and long waits.

Document prepared based on live troubleshooting session - April 18, 2026